This capture shows ICMP Echo Requests being sent out, but no Echo Replies coming back. This means this device is pinging a remote host that is not responding to ICMP — which is extremely common on the modern internet.

What can be learned from this capture?

This capture shows ICMP Echo Requests sent from my device (10.218.252.84) to a remote host (23.47.72.18). Wireshark indicates “no response found,” which means the destination host did not return ICMP Echo Replies. This behavior is normal for many internet-facing servers, which block ICMP for security reasons. The capture demonstrates how ICMP requests appear in Wireshark and highlights that the absence of replies does not necessarily indicate a network failure.

What this capture shows (packet-by-packet)

1. Source IP: 10.218.252.84

This is your device.

2. Destination IP: 23.47.72.18

This is the host you pinged.

3. Protocol: ICMP

Specifically:

- Echo (ping) request

- id=0x0001

- seq=4 and seq=5

- ttl=128

4. Info: “no response found!”

This is Wireshark saying:

“See’s my ping requests, but the target never replied.”

This is not an error — it’s normal behavior.

⭐ Why there is no response

Most public servers block ICMP for security reasons.

Reasons include:

- Preventing network mapping

- Preventing DDoS amplification

- Reducing attack surface

- Avoiding unnecessary load

So when pinging:

- Microsoft

- Google

- Akamai

- Cloudflare

- AWS

- Azure

- Many CDNs

… often there’s no reply.

This capture shows exactly that.

⭐ What this means in SOC / cybersecurity terms

This is actually a valuable learning moment, because SOC analysts need to understand:

✔ Not all hosts respond to ping

A lack of ICMP reply does not mean:

- The host is down

- The network is broken

- The packet was dropped

- Your system is misconfigured

It simply means:

“The host does not allow ICMP echo replies.”

✔ ICMP is often blocked at firewalls

Many enterprise networks block:

- ICMP Echo Reply

- ICMP Echo Request

- ICMP Timestamp

- ICMP Router Discovery

This is part of defense-in-depth.

✔ ICMP is still useful for diagnostics

Even without replies, we can still:

- See outbound ICMP

- Confirm routing

- Analyze TTL

- Inspect packet structure

- Verify interface selection

This is exactly what is happening.